7 HP ELECTRIC OUTBOARD MOTOR

Description of the Product

SEAGÖ SE7 Series is an electric outboard motor has a rated voltage is 48V, and the  maximum power of 5.1 Kw, Compared to the existing gasoline models, the electric outboard is quieter and more environmentally friendly because they run on electricity instead of fuel. Additionally, we use Permanent Magnet Synchronous Motors, which have reduced noise, extended lifespan, increased efficiency and require less maintenance.

Relevant Boat Class

Small vessels of various materials including inflatable boats, wooden boats, FRP boats, plastic boats and similar, with lengths ranging from 2 to 4 Metres, are a good fit for the electric 7-horsepower outboard motor.

Recommended Battery

For this Model, 48V 100 AH or 48V 150 AH LiFePO4 – Lithium Ferrous Phosphate battery packs can be utilized

Model Options

SE7BS – Tiller Control Short Shaft

SE7BL – Tiller Control Long Shaft

Specifications

| Product Model | SE7 Series |
| --- | --- |
| Rated Power | 5.1 kW |
| Rated Voltage | 48 V |
| Motor Type | PMSM |
| Sensor Type | Magnetic Encoder |
| Steering System | Tiller Control |
| Maximum Speed | 5000 - 5500 RPM |
| Gear Ratio | 2.08 |
| Transom Height | S:440mm /L:568mm |
| Weight {Kg} | S:30 Kg / L: 31 Kg |
| Cooling System | Water Cooled |
| Gear Position | Forward/Neutral/ Reverse |

www.seago.in       sales@seago.in      +91-8606366633

7 HP ELECTRIC OUTBOARD MOTOR